# SOFTWARE REQUIREMENTS SPECIFICATION

# Project: ASCEND

# Version: 1.0

1. Project Overview

ASCEND is a fully autonomous micro-quadrotor operating in a GNSS-denied environment.

The software stack runs on:

- Raspberry Pi 4 (Primary compute – ROS2)

- Pixhawk Flight Controller (Low-level stabilization)

- Servo-mounted TF-Luna LiDAR

- Dual IMU system

- Custom docking station (pogo-pin interface)

The objective is to build a fault-tolerant autonomous flight software stack capable of completing the defined mission tasks.

2. Mission Functional Requirements

TASK 1: Autonomous Vertical Take-Off

Requirements

SR-1.1 System shall perform pre-arm safety checks:

- IMU calibration valid

- Battery above minimum threshold

- Communication link active

SR-1.2 System shall switch to OFFBOARD mode.

SR-1.3 System shall command vertical ascent to target altitude (configurable 3–6 m).

SR-1.4 Maximum lateral drift during takeoff ≤ 0.5 m.

SR-1.5 If instability detected → immediate abort + controlled landing.

TASK 2: Hover Stability (5 minutes)

SR-2.1 Maintain altitude within ±0.2 m.

SR-2.2 Maintain XY drift within ±0.5 m radius.

SR-2.3 Hover duration ≥ 300 seconds.

SR-2.4 Use:

- VIO + IMU fusion for pose

- LiDAR for altitude correction

SR-2.5 If pose uncertainty exceeds threshold:

- Switch to safety mode

- Trigger landing

TASK 3: Controlled Landing

SR-3.1 Descent rate ≤ 0.5 m/s.

SR-3.2 Use LiDAR for ground detection < 1 m.

SR-3.3 Touchdown detection via:

- Acceleration spike

- Altitude near zero

SR-3.4 Motors auto-disarm on stable ground detection.

TASK 4: Safety Demonstration

Low Battery Return-to-Base (RTB)

SR-4.1 Continuous battery monitoring at ≥10 Hz.

SR-4.2 If battery ≤ 20%:

- Interrupt mission

- Switch to RTB state

- Navigate to dock

SR-4.3 If dock not reachable → controlled landing.

Communication Loss Handling

SR-4.4 If no heartbeat from ground station > 3 seconds:

- Switch to autonomous fallback

SR-4.5 Fallback priority:

- RTB if localization valid

- Controlled landing if localization invalid

TASK 5: Manual Docking Compatibility

SR-5.1 Software shall allow manual control override.

SR-5.2 Dock detection system shall:

- Detect dock marker

- Estimate pose relative to dock

SR-5.3 Dock alignment tolerance:

- ±2 cm lateral

- ±5° yaw

SR-5.4 Software must not resist manual landing.

TASK 6: Charging (≥5% Increase)

SR-6.1 On successful docking:

- Detect electrical contact

- Enter CHARGING state

SR-6.2 Monitor battery level.

SR-6.3 Confirm ≥5% increase before undocking.

SR-6.4 Log:

- Start time

- End time

- % increase

TASK 7: Data Transfer

SR-7.1 During flight:

- Capture and store images

- Log flight telemetry

SR-7.2 On docking:

- Automatically initiate data transfer

- Transfer:

  - Images

  - ROS logs

  - Sensor logs

SR-7.3 Verify transfer via checksum.

SR-7.4 Confirm transfer completion before charging complete.

4. Core Software Modules (To Be Implemented)

4.1 Sensor Fusion Module

Inputs:

- IMU 1

- IMU 2

- Camera VIO

- LiDAR

Requirements:

- Dual IMU failover

- Fusion latency ≤ 50 ms

- Publish pose at ≥30 Hz

4.2 LiDAR Ground Profiling Module

- Servo sweep control

- 2D ground profile generation

- Terrain feature extraction

Must improve feature matching robustness.

4.3 Mission State Machine

Mandatory states:

- INIT

- TAKEOFF

- HOVER

- MISSION

- RTB

- LAND

- DOCK_ALIGN

- DOCKED

- CHARGING

- DATA_TRANSFER

- EMERGENCY

State transitions must be deterministic.

No circular uncontrolled loops allowed.

4.4 Dock Detection Module

- Visual marker detection OR feature matching

- Pose estimation relative to dock

- Final descent guidance

Must operate at ≥15 FPS.

4.5 Logging System

Log:

- IMU data

- Battery %

- State transitions

- Pose estimates

- Failure events

Logs must be timestamped.

5. Non-Functional Requirements

Performance

- ROS2 loop ≥30 Hz

- CPU usage ≤80%

- No memory leak

Reliability

- Navigation failure rate <2%

- System must recover from IMU failure automatically

Safety

- Hard motor kill switch

- Software emergency stop topic

- Watchdog timer

6. Constraints

- No GPS allowed

- Must run on Raspberry Pi 4

- Limited compute

- No external positioning system

7. Deliverables from Software Team

- ROS2 package structure

- Node architecture diagram

- State machine diagram

- Integration with Pixhawk (MAVROS)

- Simulation validation in Gazebo

- Field test report