System Requirements & Implementation Document

Project: GPS-Denied Autonomous Hover and Auto-Land System

Platform: ArduPilot (Pixhawk) + Raspberry Pi + Intel RealSense

1. Project Overview

The objective of this project is to enable a quadcopter to maintain a stable, autonomous hover at an altitude of 3 to 4 meters in a GPS-denied environment for exactly 60 seconds, followed by an autonomous safe landing. Positioning data will be derived locally using visual odometry and sensor fusion.

2. Hardware Architecture & Wiring

This system relies on a Companion Computer (CC) processing high-bandwidth camera data and feeding position estimates to the Flight Controller (FC).

- Flight Controller (FC): Pixhawk 2.4.8 running ArduPilot (firmware supporting EKF3).

- Companion Computer (CC): Raspberry Pi 4B.

- Vision Sensor: Intel RealSense D455.

- Physical Interfaces:

  - CC to FC: Raspberry Pi GPIO pins (UART TX/RX) connected to the Pixhawk's Telem2 port.

  - Baud Rate: 921600 bps.

  - Camera to CC: USB 3.0 connection (mandatory for D455 depth/IMU bandwidth).

3. System Data Flow (Architecture Pipeline)

Based on the system design, the processing pipeline is distributed between the RPi and the Pixhawk:

- D455 Camera: Captures Stereo IR/RGB and internal IMU data.

- Optical Flow / Visual Odometry (on RPi): Computes pixel displacement between frames.

- EKF Fusion (on RPi): Fuses the optical flow vectors with the D455's high-frequency IMU data.

- Position Estimate (on RPi): Generates local coordinate outputs $(X, Y, Z)$ and attitude estimates.

- MAVLink Bridge (RPi $\rightarrow$ Pixhawk): Packages the data into a VISION_POSITION_ESTIMATE message and transmits it via Telem2.

- PID Controller (on Pixhawk): ArduPilot's EKF3 ingests the vision data and Barometer data to feed the internal PID loops.

- Motor Commands: The Pixhawk adjusts the ESCs/motors to maintain the hover.

4. Software Functional Requirements

The Python software stack running on the Raspberry Pi must accomplish the following three distinct tasks simultaneously:

Task 1: Hardware Interfacing & Initialization

- MAVLink Connection: Initialize a pymavlink connection on the defined serial port (e.g., /dev/serial0) at 921600 baud. The script must pause and wait for a valid heartbeat from the Pixhawk before proceeding.

- RealSense Pipeline: Initialize the pyrealsense2 pipeline. It must enable the depth stream, infrared/RGB streams (for feature tracking), and the IMU streams (accelerometer and gyroscope).

Task 2: Flight Logic & Autonomous Hover (Round 1)

- Continuous Odometry: Calculate the local $(X, Y, Z)$ position at a minimum of 20Hz.

- Flight Mode Monitoring: The script must continuously monitor the Pixhawk's current flight mode.

- The Hover Trigger: * The pilot will manually take off in Stabilize mode to a target altitude of 3 to 4 meters.

  - The pilot will manually switch the radio transmitter to Loiter mode.

  - Upon detecting the switch to LOITER, the script must start a precise 60-second timer.

  - During these 60 seconds, the script must ensure uninterrupted transmission of the VISION_POSITION_ESTIMATE messages so the drone holds its exact position.

Task 3: Autonomous Safe Landing

- Mode Switching: Exactly 60 seconds after entering LOITER mode, the Python script must issue a MAVLink command to change the flight mode to Land (MAV_CMD_NAV_LAND).

- Descent Stability: While the drone is descending, the script must continue sending the VISION_POSITION_ESTIMATE data. This ensures the drone uses the camera to land straight down without horizontal drift.

5. Required ArduPilot Configuration (Mission Planner)

To ensure the Pixhawk's Extended Kalman Filter (EKF3) accepts the MAVLink messages from the Raspberry Pi, the following parameters must be written to the FC:

| Parameter | Value | Purpose |
| --- | --- | --- |
| SERIAL2_PROTOCOL | 2 | Enables MAVLink 2 on the Telem2 port. |
| SERIAL2_BAUD | 921 | Matches the RPi's 921600 baud rate. |
| EK3_SRC1_POSXY | 6 | Tells EKF3 to use "External Navigation" (the camera) for $X, Y$. |
| EK3_SRC1_POSZ | 1 | Tells EKF3 to use the Barometer for vertical $Z$ altitude. |
| EK3_SRC1_YAW | 6 | Uses the camera's odometry for heading/yaw calculation. |
| VISO_TYPE | 1 | Enables the MAVLink visual odometry listener. |
| COMPASS_USE | 0 | Disables the compass to prevent magnetic interference indoors. |

Flow diagram architecture: